\magnification = \magstep1

\nopagenumbers

\centerline{\bf Figure 1. Distinguishable and Indistinguishable Genotypes }

\bigskip

When the wild-type and mutant homozygous genotypes are

symmetric with respect to nearest-neighbor thermodynamics,

their normalized melting curves are indistinguishable.

This figure shows three replicate normalized melting curves

of each of the three genotypes of the H63D (Hemochromatosis)

mutation, which obeys this symmetry. Six curves representing

the wild-type and mutant homozygous genotypes appear as

one curve, while the normalized melting curves of the heterozygote

appear as another easily distinguishable curve.

\vfill\eject

\centerline{\bf Figure 2. Theoretical duplex melting curves, superpositions and differences }

\bigskip

After amplification of heterozygous DNA (or a mixture of any

two genotypes of a biallelic SNP) two types of forward strands

reassociate independently with two types of reverse strand,

forming two species of homoduplex and two species of heteroduplex.

A fluorescent dye gives a signal proportional to the quantity of

DNA which is in its double-stranded state, which depends on

the temperature and the thermodynamic stability of each

duplex type.  The overall fluorescence vs. temperature `melting

curve' is a superposition of the melting curves of the individual

duplexes in proportion to their relative concentrations.

\bigskip

This figure gives the melting curves for the four duplex species

of heterozygous or mixed genotype H63D amplicons as predicted

by nearest-neighbor theory model with parameters described

in (SL match and CC,GG). The wild-type and mutant homozygous

curves are identical. It also shows the theoretical heterozygote

melting curve obtained as the equally weighted average of

the four duplex curves. In the nearest-neighbor symmetric case,

the difference between any genotype mixture melting curves is

proportional to a standard difference curve which is the difference

between the average homoduplex curve and the average heteroduplex curve,.

by a factor equal to the absolute difference in heteroduplex content.

Thus,  the difference between the

heterozygote melting curve and the overlapping homoduplex curves,

which is also shown, is equal to the standard difference curve

scaled by ${1 \over 2}$, the heteroduplex fraction of an amplified

heterozygote.

\vfill \eject

\centerline{\bf Figure 3. Theoretical Dependence Of Heteroduplex Content On Mixture Fraction }

\bigskip

This figure shows the magnitude of the heteroduplex content differences

amomg mixtures obtained by adding wild-type DNA to DNA of each of the

three genotypes of a bi-allelic SNP (W=wild-type, M=mutant homozygous,

H=heterozygous) in the relative proportions $x$ of added wild-type

(the `mixture fraction') to $1-x$ of the varied genotype. We assume

that strands of different types associate independently after either

PCR amplification is completed or a subsequent dissociation by heating

and reassociation by cooling have been performed. We seek the

mixture fraction providing the greatest separation of heteroduplex

content between the resulting mixtures, represented by the highest

point on the lowest among the three graphs. This point which is

marked at $x={1 \over 7}$, gives the optimal mixture fraction.

\vfill \eject

\centerline{\bf Figure 4. Experimental Melting Curve Separation Dependence On Mixture Fraction }

\bigskip

This figure shows the mean and standard deviation (error bars) of

the experimentally measured maximum melting curve separation

between replicates of mixtures of wild-type DNA with either

mutant heterozygous or heterozygous samples, and the mean of the

wild-type replicates, as a function of the mixture fraction. The

values are normalized by a factor which makes the difference for an

unmixed heterozygous sample equal to ${1 \over 2}$ to agree with the

theoretical prediction in terms of heteroduplex content (superimposed).

\vfill\eject

\centerline{\bf Figure 5a. Genotype Separation At Optimal Mixture Fraction 4/28 }

\bigskip

This figure shows the normalized melting

curves for three replicates of optimal mixtures of

wild-type DNA and each genotype of sample. As predicted, the

mutant homozygous curves lie equidistant from the wild-type

curves and the barely altered heterozygous curves.

\vfill\eject

\centerline{\bf Figure 5b.  Genotype Separation At Non-Optimal Mixture Fraction 9/28 }

\bigskip

\centerline{\bf Figure 5c.  Genotype Separation At Non-Optimal Mixture Fraction 14/28 }

\bigskip

\centerline{\bf Figure 5d.  Genotype Separation At Non-Optimal Mixture Fraction 19/28 }

\bigskip

These figures show the normalized

melting curves for three replicates of other-than-optimal mixtures

of wild-type DNA and each genotype of sample.

In Figure 5b, with mixture fraction is ${9 \over 28}$ near $x={1 \over 3}$,

the mutant homozygous curves and the heterozygous curves have

become indistinguishable, as predicted.

In Figure 5c, with mixture fraction is ${14 \over 28}$,

i.e., wild-type and sample DNA mixed in equal

proportions), the mutant homozygous curves and the heterozygous curves

are also difficult to distinguish.

In Figure 5d, with mixture fraction is ${19 \over 28}$

near the upper local maximum $x={2 \over 3}$, the mutant homozygous

curves and the

heterozygous curves have reversed position and are

distinguishable, but not nearly as well as at the optimal mixture fraction.

\vfill\eject

\centerline{\bf Figure 6a. Experimental Difference Curves Of Pure Genotypes }

\bigskip

\centerline{\bf Figure 6b. Experimental Difference Curve Of Optimally Mixed Genotypes }

\bigskip

These figures show the difference of normalized melting

curves of unmixed and optimally mixed samples of

the three genotypes and the mean of the wild-type

normalized melting curve replicates. The shape

of the curves reflects the predicted standard

difference curve shape, and the amplitude agrees

with the predicted dependence on heteroduplex

content difference.

\vfill\eject

\centerline{\bf Figure 7a. Raw TGCE Duplex Arrival Frequency Data }

\bigskip

\centerline{\bf Figure 7b.  Normalized TGCE Duplex Arrival Frequency Data }

\bigskip

These figures show the raw and normalized measurements of

duplex arrival frequency (counts per frame) for three

replicates of each genotype and wild-type DNA with mixture

fraction $x={9 \over 28}$. After normalization by shifting

and scaling the largest peaks to a common location and

height, the common heteroduplex content of the mutant

homozygous and heterozygous sample mixtures for this

mixture fraction are visible from the common first and

second heteroduplex peak heights.

\vfill\eject

\centerline{\bf Figure 8.  Heteroduplex Content Dependence On Mixture Fraction From TGCE }

\bigskip

This figure shows the mean and standard deviation (error bars) of

the experimentally derived heteroduplex content of

replicates of mixtures of wild-type DNA with either mutant heterozygous

or heterozygous samples, as a function of the mixture fraction. The

values are adjusted by a factor which makes the value for

control heterozygous samples equal to ${1 \over 2}$ to agree with

the theoretical prediction of heteroduplex content

(superimposed).

\end